Electrify Gas Lamp, Additional  Installation Comments for Homeowners and Contractors

ECC Application

- Please Note: Complete this ECC  Form when converting your existing Gas Lamp to                                                                                                      Electric and check one of the OPC approved conversion kits:

http://oldeportcove.com/wp-content/uploads/2017/09/ECC-Gas-Lamp-Change.pdf

Turn Off Gas Lamp House Valve

- Turn the lamp gas tubing (line) valve off from the house, usually 90-deg on-off valve turn.

The valve may be inside garage, inside house closet or cabinet, in crawl space or outside by gas meter. Important: this valve should be padlocked or locked and/or “Red Tagged” so the valve will not be turned on again. You could also plug (cap) the gas line next to the valve.

Remove Lamp

- After you remove the three lamp cap set screws the lamp may not lift up to access and disconnect the gas fittings. Then you need to dig up the lamp pole to free up the gas tubing so the lamp will slide up from the pole. Note: The pole may be set in concrete. Note: The set screws may be stuck and will need to be drilled out. Small bolts (1/4 In 20 Threads/In ½-In long) could be used in place of the set screws

Plug Gas Line

- Also suggest you install a 1/4 In Flair Plug to cap off the gas tubing at the top of pole.

Link instructions with videos: https://www.charm-lite.com/index.php?main_page=page_2

All new gas fittings should be leak tested by applying soapy water to the fittings.

Amazon ¼ In Flare Plug $1.59:

https://www.amazon.com/Parker-Hannifin-639F-4-Degree-Fitting/dp/B008FMUWSA/ref=sr_1_2?keywords=1%2F4+in.+flair+Flare+Plug&qid=1580776930&sr=8-2

Remove Gas Stem

4)  Your $320 Gothic lamp is used with the electric kit; however the gas lamp stem must be removed to

Install the electric stem. First remove the lamp bottom cap to expose the gas valve. Unscrew the top gas stem by the air holes with a screw drive inside the air holes or grip the stem by the air holes with vice grips. Righty-tighty–lefty-loosey. The lower stem may be hard to free up, use a 1-in socket with breaker bar or ratchet inside the lamp stem and a vice grip on the outside gas valve. Or maybe clamp the gas valve in a vice. Be careful not to break your $320 fragile Gothic Lamp.

Outside Transformer

- If you install your transformer on an outside post or outside wall, use a plastic electrical cover box to weatherize the transformer.

Do Not Install Dusk to Dawn Controller

6)   The OPC Covenants require lamps on 24-7 so do not install a Dusk to Dawn Controller.

Disclaimer,  Gas and Electrical Codes

7)   The Homeowner is solely responsible for ensuring that installation meets any applicable federal, state, or local installation gas and electrical codes. Working with gas and electricity is extremely dangerous. This document was prepared to help installation, and be sure to follow the manufactures’ installation directions.

Please e-mail oldeportcove@hotmail.com with helpful installation comments.

C;\oldeportcove\Electrify-gas lamps\ Electrify Gas Lamps Comments.doc